10/27/2024

Use this document to estimate power consumption!

Overview

- Currently, microcontroller is powered by 3 AA batteries

  - This is a total of 4.5V

- The microcontroller needs 3.3V to operate

  - The ESP8266 Thing Dev has a built-in voltage regulator, so it can be fed up to 6V and still operate correctly

- The 1.5V batteries will begin to drop in voltage quickly

  - How quickly depends on how much current the microcontroller is drawing

- Standard capacity of AA batteries is 2000mAh (regardless of how many are in series)

  - Average current draw of Thing Dev is 80mA

  - This results in a theoretical lifetime of 25 hours

  - The true hours of reliable operation are lower because the voltage supplied to the microcontroller would drop below 3.3V before that

- Transmitting data over Wi-Fi will require more current than 80mA (probably about 150mA)

- Deep sleep should be used to prolong the battery life

  - Microcontroller only uses 20µA while in deep sleep

  - Needs to reconnect to WiFi when waking up from deep sleep

- Say deep sleep is used and Thing Dev is brought out of sleep every 15 seconds

  - 4,320 wake ups each day (20 seconds total between sleeping and active)

  - Say it takes 5 seconds to connect to Wifi, read all sensors and transmit data

  - 4,320 ×5=21,600seconds = 360 minutes=6 hours each day

- NEED TO SLEEP LONGER THAN 15 SECONDS BETWEEN MEASUREMENTS

Using Deep Sleep

- ESP.deepSleep(time)

  - Time specified in MICROSECONDS

- When microcontroller comes out of deep sleep it runs the setup block

  - There should be no need for the loop block

- Reset pin needs to be connected to GPIO 16

  - CONNECT RST PIN TO GPIO 16 ONLY AFTER UPLOADING THE CODE

  - ESP 8266 Thing Dev has a built-in SLEEP-EN jumper that achieves this

  - Solder 2-pin male headers and then just slide the 2-pin jumper on and off

Vin vs. 5V pins

- In the breadboard design, all of the sensors and the ADC are powered by Vin

- Vin is directly connected to whatever is supplying the board with power

- 2400 mAh Batteries

- Average current draw of Thing Dev is 80mA

- Roughly 30 hours of battery lifetime

- Assume 15 seconds each time to read from sensors, connect to wifi, initialize SD, and other operations. This means 720 wake ups a day.

- So roughly 720 wakeups * 15 seconds in operation = 10800 sec = 180 mins = 3 hours a day. 30 hours total lifetime / 3 hour a day = 10 days!